Contest:

Location of MeasureStations

|  | Station 1 | Station 2 | Station 3 |
| --- | --- | --- | --- |
| Horizontal Rectangle | 90º | 90º | 90º |
| Vertical Rectangle | 100º | 100º | 100º |
| HorizontalAngle to the left NeighbourStation | º | º | º |
| HorizontalAngle to the right NeighbourStation | - º | - º | - º |
| VerticalAngle to the LaunchPosition | º | º | º |
| Distance to the LaunchPosition | 300m | 300m | 300m |

M1

Drawing of LaunchSite

S

M2

M3

m

a = -   º

m

m

a =    º

a = -   º

a =    º

a = -   º

a =    º

g =   º

g =   º

g =   º

| Definitions |  |  |
| --- | --- | --- |
| Horizontal/Vertical Rectangles | - | The Programm knows only 90 or 100 º Rectangles. |
| HorizontalAngle to the NeighbourStations | - | The Numbers of Stations are very importent for a proper Calculation. Please get for the Stations Numbers in such Order, that they are encrease continuisely in a direction against the direction of the nomal way of ClockPointers - how it shows the predrawen example in the page before. So the left NeighbourStation of Station 3 is the Station 2 and the right NeighbourStation of Station 3 is the always the Station 1. When a NeighbourStation - looking at the LaunchPosition - is on the LEFT, the AngleValue is POSITIVE - otherwise, when a NeighbourStation - looking at the LaunchPosition - is on the RIGHT - the AngleValue is NEGATIVE. |
| VerticalAngle to the LaunchPosition | - | This Angle is NEGATIVE, when the LaunchPosition is LOWER than the Position of the Station. |
| Distance to the LaunchPosition | - | is the shortest Line between a Station and LaunchPosition in Meters. |

Notices

You can set a Measure Station in a very short Distance from the LaunchPosition. It is not necessary that LaunchPosition is in the circle of MeasureStations. Also, the REALLY LaunchPosition is not importent - importent is just, that every Station is looking on the same Point while measuring angles for Calculating the Positions of MeasureStations.

Excuses

I know - my English is not Perfect. - Make ist better where ever You think that is necessary.